A) VSÁDZKOVÝ BIOREAKTOR NA KULTIVÁCIU BIOMASY

Dosadením do rovnice (2.3) sa získa vzťah:

Obr. 1 Príklad sigmoidnej krivky pre periodický kultivačný proces.

Obr. 2

B) „FED-BATCH“ BIOREAKTOR NA KULTIVÁCIU BIOMASY

Fed-batch culture is, in the broadest sense, defined as an operational technique in biotechnological processes where one or more nutrients (substrates) are fed (supplied) to the bioreactor during cultivation and in which the product(s) remain in the bioreactor until the end of the run. An alternative description of the method is that of a culture in which "a base medium supports initial cell culture and a feed medium is added to prevent nutrient depletion".

It is also a type of semi-batch culture.

In some cases, all the nutrients are fed into the bioreactor. The advantage of the fed-batch culture is that one can control concentration of fed-substrate in the culture liquid at arbitrarily desired levels (in many cases, at low levels).

Obr. 3 Fed Batch biorekator.

Generally speaking, fed-batch culture is superior to conventional batch culture when controlling concentrations of a nutrient (or nutrients) affects the yield or productivity of the desired metabolite.

Obr. 4 The graph shows the principle of a substrate limited fed-batch cultivation with an initial batch phase. After consumption of the initial substrate a continuous and constant feed of the substrate is started.

The types of bioprocesses for which fed-batch culture is effective can be summarized as follows:

1. Substrate inhibition

Nutrients such as methanol, ethanol, acetic acid, and aromatic compounds inhibit the growth of microorganisms even at relatively low concentrations. By adding such substrates properly lag-time can be shortened and the inhibition of the cell growth markedly reduced.

2. High cell density (High cell concentration)

In a batch culture, to achieve very high cell concentrations, e.g. 50-100 g of dry cells/L, high initial concentrations of the nutrients in the medium are needed. At such high concentrations, the nutrients become inhibitory, even though they have no such effect at the normal concentrations used in batch cultures.

3. Glucose effect (Crabtree effect)

In the production of baker's yeast from malt wort or molasses it has been recognized since early 1900s that ethanol is produced even in the presence of sufficient dissolved oxygen (DO) if an excess of sugar is present in the culture liquid. Ethanol is a main cause of low cell yield. Aerobic ethanol formation in the presence of glucose concentration is known as glucose effect or Crabtree effect. To reduce this effect, a fed-batch process is generally employed for baker's yeast production. In aerobic cultures of Escherichia coli and Bacillus subtilis, organic acids such as acetic acid, (and in lesser amounts, lactic acid and formic acid), are produced as byproducts when sugar concentration is high, and these acids inhibit cell growth as well as show deteriorating effect on the metabolic activities. The formation of these acids are called bacterial Crabtree effects.

4. Catabolite repression

When a microorganism is provided with a rapidly metabolizable carbon-energy source such as glucose, the resulting increase in the intracellular concentration of ATP leads to the repression of enzyme(s) biosynthesis, thus causing a slower metabolization of the energy source. This phenomenon is known as catabolite repression. Many enzymes, especially those involved in catabolic pathways, are subject to this repressive regulation. A powerful method of overcoming the catabolite repression in the enzyme biosynthesis is a fed-batch culture in which glucose concentration in the culture liquid is kept low, where growth is restricted, and the enzyme biosynthesis is derepressed. Slow feeding of glucose in penicillin fermentation by Penicillium chrysogenum is a classical example in the category.

5. Auxotrophic mutants

Auxotrophy is the inability of an organism to synthesize a particular organic compound required for its growth (as defined by IUPAC). An auxotroph is an organism that displays this characteristic; auxotrophic is the corresponding adjective. Auxotrophy is the opposite of prototrophy, which is characterized by the ability to synthesize all the compounds needed for growth.

In a microbial process employing an auxotrophic mutant (nutritionally requiring mutant), excess supply of the required nutrient results in abundant cell growth with little accumulation of the desired metabolite due to feedback inhibition and /or end-product repression. Starvation of the required nutrient, however, lowers cell growth as well as the overall production of the desired metabolite, as the production rate is usually proportional to the cell concentration. In such a bioprocess, the accumulation of the desired metabolite can be maximized by growing the mutant on a limited amount of the required nutrient. To cultivate the mutant on a low concentration of the required nutrient, it is fed to the batch culture at a controlled rate. This technique is often used in industrial amino acid productions with the auxotrophic mutants. An example is lysine production with homoserine- or threonine/methionine-requiring mutant of Corynebacterium glutamicum being lacking for homoserine dehydrogenase gene.

6. Expression control of a gene who has repressible promoter in recombinant cells

Transcription of a gene having a repressible promoter in the upstream of open reading frame is repressed by combination of so-called holo-repressor with operator region of DNA. When a specified chemical compound exists in the culture liquid, the compound (or its metabolite) in the cells combines as co-repressor with an apo-repressor ( a kind of transcription factor) to form the holo-repressor. Therefore, the concentration of the compound in the culture liquid should be controlled at as low level as possible while the level supports sufficient cell growth. To the end, a fed-batch culture is powerful technique. Examples of the repressible promoter are trp promoter and phoA promoter.

7. Extension of operation time, supplement of water lost by evaporation, and decreasing viscosity of culture broth

High cell-density culture

The fed-batch strategy is typically used in bio-industrial processes to reach a high cell density. Mostly the feed solution is highly concentrated to avoid dilution of the bioreactor. Production of heterologous proteins by fed-batch cultures of recombinant microorganisms have been extensively studied.

The controlled addition of the nutrient directly affects the growth rate of the culture and helps to avoid overflow metabolism (formation of side metabolites, such as acetate for Escherichia coli, lactic acid in mammalian cell cultures, ethanol in Saccharomyces cerevisiae), oxygen limitation (anaerobiosis).

Constantly-fed-batch culture

The simplest fed-batch culture is the one in which the feed rate of a growth-limiting substrate is constant, i.e. the feed rate is invariant during the culture. This case is shown in the graph(here the culture volume is variable). This type of the fed-batch culture is named constantly-fed-batch culture (CFBC), and is well established mathematically and experimentally. In the CFBC, both cases of fixed-volume CFBC and variable-volume CFBC were studied.

Exponentially-fed-batch culture

Under ideal condition, cells grow exponentially. If the feed rate of the growth-limiting substrate is increased in proportion to the exponential growth rate of the cells, it is possible to maintain the cells' specific growth rate for a long time while keeping the substrate concentration in the culture liquid at a constant level. The required feed rate (volumetric or mass) must be increased exponentially with time so that this mode of fed-batch culture is called exponentially-fed-batch culture (EFBC).

Substrate limitation offers the possibility to control the reaction rates to avoid technological limitations connected to the cooling of the reactor and oxygen transfer. Substrate limitation also allows the metabolic control, to avoid osmotic effects, catabolite repression and overflow metabolism of side products.[16][17][18]

B) KONTINUÁLNÝ BIOREAKTOR NA KULTIVÁCIU BIOMASY

CHEMOSTAT

Obr. 5 Stirred bioreactor operated as a chemostat, with a continuous inflow (the feed) and outflow (the effluent). The rate of medium flow is controlled to keep the culture volume constant.

A chemostat (from chemical environment is static) is a bioreactor to which fresh medium is continuously added, while culture liquid containing left over nutrients, metabolic end products and microorganisms are continuously removed at the same rate to keep the culture volume constant. By changing the rate with which medium is added to the bioreactor the specific growth rate of the microorganism can be easily controlled within limits.

STEADY STATE

One of the most important features of chemostats is that microorganisms can be grown in a physiological steady state under constant environmental conditions. In this steady state, growth occurs at a constant specific growth rate and all culture parameters remain constant (culture volume, dissolved oxygen concentration, nutrient and product concentrations, pH, cell density, etc.). In addition, environmental conditions can be controlled by the experimenter. Microorganisms growing in chemostats usually reach a steady state because of a negative feedback between growth rate and nutrient consumption: if a low number of cells are present in the bioreactor, the cells can grow at growth rates higher than the dilution rate as they consume little nutrient so growth is less limited by the addition of limiting nutrient with the inflowing fresh medium. The limiting nutrient is a nutrient essential for growth, present in the medium at a limiting concentration (all other nutrients are usually supplied in surplus). However, the higher the number of cells becomes, the more nutrient is consumed, lowering the concentration of the limiting nutrient. In turn, this will reduce the specific growth rate of the cells which will lead to a decline in the number of cells as they keep being removed from the system with the outflow. This results in a steady state. Due to the self-regulation, the steady state is stable. This enables the experimenter to control the specific growth rate of the microorganisms by changing the speed of the pump feeding fresh medium into the vessel.

WELL-MIXED

Another important feature of chemostats and other continuous culture systems is that they are well-mixed so that environmental conditions are homogenous or uniform and microorganisms are randomly dispersed and encounter each other randomly. Therefore, competition and other interactions in the chemostat are global, in contrast to biofilms.

DILUTION RATE

The rate of nutrient exchange is expressed as the dilution rate(D).At steady state, the specific growth rate (μ) of the micro-organism is equal to the dilution rate (D). The dilution rate is defined as the flow of medium per time (F) over the volume of culture (V) in the bioreactor

INDUSTRY

Chemostats are frequently used in the industrial manufacturing of ethanol. In this case, several chemostats are used in series, each maintained at decreasing sugar concentrations.[citation needed]

CONCERNS

- Foaming results in overflow with the volume of liquid not exactly constant.

- Some very fragile cells are ruptured during agitation and aeration.

- Cells may grow on the walls or adhere to other surfaces,[16] which may be overcome by treating the glass walls of the vessel with a silane to render them hydrophobic. However, cells will be selected for attachment to the walls since those that do will not be removed from the system. Those bacteria that stick firmly to the walls forming a biofilm are difficult to study under chemostat conditions.

- Mixing may not truly be uniform, upsetting the "static" property of the chemostat.

- Dripping the media into the chamber actually results in small pulses of nutrients and thus oscillations in concentrations, again upsetting the "static" property of the chemostat.

- Bacteria travel upstream quite easily. They will reach the reservoir of sterile medium quickly unless the liquid path is interrupted by an air break in which the medium falls in drops through air.

Continuous efforts to remedy each defect lead to variations on the basic chemostat quite regularly. Examples in the literature are numerous.

- Antifoaming agents are used to suppress foaming.

- Agitation and aeration can be done gently.

- Many approaches have been taken to reduce wall growth

- Various applications use paddles, bubbling, or other mechanisms for mixing

- Dripping can be made less drastic with smaller droplets and larger vessel volumes

- Many improvements target the threat of contamination

TEÓRIA CHEMOSTATU

Obr. 6

Obr. 7

Obr. 8

Obr. 9

(3.40)

Obr. 10

Obr. 11

DIFFERENCE BETWEEN BATCH AND CONTINUOUS CULTURE

Microorganisms such as bacteria and fungi are very beneficial to various types of industries. For industrial usage, microorganisms should be grown in large scale during the fermentation process in order to extract the necessary products resulting from the microbial metabolism. A special apparatus called industrial fermenter is used to cultivate and maintain microbial biomass. It is a large vessel designed to provide space and necessary requirements for microbial growth and metabolism. There are two types of industrial fermentation cultures commonly adopted in industries named batch culture and continuous culture. The key difference between batch culture and continuous culture is that batch culture is a technique used to grow microorganisms under limited nutrient availability in a closed system while continuous culture is a technique used to grow microorganisms under optimum and continual supply of nutrients in an open system in industries.

What is a Batch Culture?

Batch culture is a technique which grows microorganisms in a closed system where a limited amount of nutrients are supplied at the beginning. This is the commonest technique adopted in industries to make useful products using microorganisms such as bacteria and fungi. Microbe which grows in the fermenter ferments the nutrients.

Fermentation is a breaking down process of carbohydrates into alcohols and acids by microorganisms under anoxic conditions. In batch culture technique, nutrients are provided at the beginning and the particular microorganism is inoculated into the fermenter. The fermenter is closed and the temperature and pH are maintained for the growth of microorganisms. Microorganism grows inside and utilizes the provided nutrients and other conditions. With time, nutrients become limited and the environmental conditions change within the fermenter. Hence, microbial growth shows distinct four stages such as lag phase, log phase, stationary phase, and death phase. At the end of the fermentation, the process is stopped and useful products are extracted and purified. The fermenter is washed and sterilized before using for another batch culture.

The specialty of the batch culture technique is, this is run under limited amounts of nutrients and for a certain time period. The fermenter setup is easy to make and handle. Environmental conditions inside the fermenter vary with time. However, required temperature, pH conditions, stirring, pressure, etc. are properly maintained to achieve successful product formation.

Batch culture technique is widely used for the purification of secondary metabolites such as antibiotics, pigments, etc. This technique is not suitable for the production of primary metabolites and products which are associated with growth.

What is a Continuous Culture?

Continuous culture is another technique which grows useful microorganisms. It aims to maintain a continuously growing microbial culture at exponential phase. It can be achieved by supplying fresh nutrients continually, removing accumulated waste and products at the same rate and keeping other conditions at the optimum values. It is done inside a special chamber called chemostat. Fresh medium is added continuously from one end while metabolic products are continually extracted from the other end of the chemostat to keep the culture volume at a constant level.

Continuous culture is used in industries when it is required to extract useful primary metabolites such as amino acids, organic acids, etc. from the microorganisms. Primary metabolites are produced at the highest rate when the microorganisms are at their exponential phase. Hence continuous culture always aims to maintain the microbial biomass at the log phase. It is done by monitoring the process continually and controlling the system.

Obr. 12 Continuous Culture in Chemostat

Summary – Batch vs Continuous Culture

Batch culture and continuous culture are two types of techniques employed to cultivate microorganisms in large scale for industrial and other purposes. In batch culture, microorganisms are provided with nutrients at the beginning and grown. When microbes utilize the available nutrients, nutrients become limited after certain time period. Microorganisms grow via lag, log, stationary and death phases. The fermentation process is carried out batch-wise in batch culture technique. After each and every batch, fermenter is cleaned and used freshly for the next batch. In continuous culture, microorganisms are provided with adequate levels of fresh nutrients continually to always maintain the microbes at log phase to extract primary metabolites of the microorganisms. The volume of the continuous culture is maintained at a constant value by adding fresh nutrients and removing products at the same rate without stopping the process. Batch culture is needed comparatively a large closed fermenter while continuous culture is needed a small open fermenter. This is the difference between batch and continuous culture.

TURBIDOSTAT

A turbidostat is a continuous microbiological culture device, similar to a chemostat or an auxostat, which has feedback between the turbidity of the culture vessel and the dilution rate.

The theoretical relationship between growth in a chemostat and growth in a turbidostat is somewhat complex, in part because they are similar.

A chemostat has a fixed volume and flow rate, and thus a fixed dilution rate.

A turbidostat dynamically adjusts the flow rate (and therefore the dilution rate) to make the turbidity constant.

At equilibrium, operation of both the chemostat and turbidostat are identical.

It is only when classical chemostat assumptions are violated (for instance, out of equilibrium; or the cells are mutating) that a turbidostat is functionally different.

One case may be while cells are growing at their maximum growth rate, in which case it is difficult to set a chemostat to the appropriate constant dilution rate.

While most turbidostats use a spectrophotometer/turbidometer to measure the optical density for control purposes, there exist other methods, such as dielectric permittivity.

Obr. 13 Príklad zapojenia turbidostatu.

Schematic diagram of turbidostat. It uses the optical density measurements to modulate how much media is pumped into the culture vessel every minute to maintain a constant cell density. Through this method of cell density control, we can measure how fast the cells are growing and thus gain an understanding for relative fitness increases thorough out the course of an experiment.

AUXOSTAT

Obr. 14

An auxostat is a continuous culture device which, while in operation, uses feedback from a measurement taken on the growth chamber to control the media flow rate, maintaining the measurement at a constant.

Auxo was the Greek goddess of spring growth, and as a prefix represents nutrients.

However, the most typical auxostats are pH-auxostats, with feedback between the growth rate and a pH meter.

Other auxostats may measure oxygen tension, ethanol concentrations, and sugar concentrations.

RETENTOSTAT

A form of chemostat in which culture liquid is removed from the bioreactor but a filter retains the biomass

Obr. 15

CONTINUOUS STIRRED-TANK REACTOR

Obecná definícícia používaná v chemickom inžinierstve pre akékoľvek využitie reaktora, napr. pre chémiu, biochémiu,....

The continuous flow stirred-tank reactor (CSTR), also known as vat- or backmix reactor, is a common ideal reactor type in chemical engineering.

A CSTR often refers to a model used to estimate the key unit operation variables when using a continuous agitated-tank reactor to reach a specified output. The mathematical model works for all fluids: liquids, gases, and slurries.

Obr. 16

The behavior of a CSTR is often approximated or modeled by that of a Continuous Ideally Stirred-Tank Reactor (CISTR). All calculations performed with CISTRs assume perfect mixing. In a perfectly mixed reactor, the output composition is identical to composition of the material inside the reactor, which is a function of residence time and rate of reaction. If the residence time is 5-10 times the mixing time, this approximation is valid for engineering purposes. The CISTR model is often used to simplify engineering calculations and can be used to describe research reactors. In practice it can only be approached, in particular in industrial size reactors.

Assume:

perfect or ideal mixing, as stated above.

Integral mass balance on number of moles Ni of species i in a reactor of volume V.

where Fio is the molar flow rate inlet of species i, Fi the molar flow rate outlet, and νi stoichiometric coefficient.  The reaction rate, ri, is generally dependent on the reactant concentration and the rate constant (k). The rate constant can be determined by using a known empirical reaction rates that is adjusted for temperature using the Arrhenius temperature dependence. Generally, as the temperature increases so does the rate at which the reaction occurs. Residence time τ is the average amount of time a discrete quantity of reagent spends inside the tank.

Assume:

- constant density (valid for most liquids; valid for gases only if there is no net change in the number of moles or drastic temperature change)

- isothermal conditions, or constant temperature (k is constant)

- steady state

- single, irreversible reaction (νA = -1)

- first-order reaction (r = kCA)

- A → products

NA = CA.V (where CA is the concentration of species A, V is the volume of the reactor, NA is the number of moles of species A)

The values of the variables, outlet concentration and residence time, are major design criteria.

To model systems that do not obey the assumptions of constant temperature and a single reaction, additional dependent variables must be considered. If the system is considered to be in unsteady-state, a differential equation or a system of coupled differential equations must be solved.

CSTR's are known to be one of the systems which exhibit complex behavior such as steady-state multiplicity, limit cycles and chaos.

Chemical reactors often have significant heat effects, so it is important to be able to add or remove heat from them. In a CSTR (continuous stirred tank reactor) the heat is added or removed by virtue of the temperature difference between a jacket fluid and the reactor fluid. Often, the heat transfer fluid is pumped through agitation nozzle that circulates the fluid through the jacket at a high velocity. The reactant conversion in a chemical reactor is a function of a residence time or its inverse, the space velocity. For a CSTR, the product concentration can be controlled by manipulating the feed flow rate, which changes the residence time for a constant chemical reactor. Occasionally the term "continuous" is misinterpreted as a modifier for "stirred", as in 'continuously stirred'. This misinterpretation is especially prevalent in the civil engineering literature. As explained in the article,"continuous" means 'continuous-flow' — and hence these devices are sometimes called, in full, continuous-flow stirred-tank reactors (CFSTR's).

OTÁZKY NA SKÚŠKU:

1. Čo je chemostat. teória chemostatu, definícia narieďovacieho pomeru a predpoklady pre činnosť chemostatu.

2. Vysvetlite bilanciu na mikrobiálnu hmotu, substrát, produkt, základné bilančné rovnice , bilancia na biomasu, substát a produkt v chemostate, odklon od monodeovej teórie.

3. Turbistat, princíp činnosti.