In this chapter we introduced the idea of growth reaction to characterize material balance associated with cell growth. We defined a chemical formula which represents about 95% of the dry matter of biomass. We also defined yield which enables us to derive usful engineering information from literature articles that report cell yield. To carry out material balances around a fermentor requires, in general, respiratory quotient values and one other measurement.

CELL COMPOSITION

Cells primarily contain water!

Typically 70% of cell mass is water and the remaining is dry matter.

Therefore it is conventional to express cell composition on a dry basis.

The microorganism Eschericia coli is widely used in genetic engineering.

Typical elements found in Eschericia coli are given in Table 1.

Table 1 Elemental Composition of E. coli  (after Stanier et al)

| Element | % Dry Weight |
| --- | --- |
| C | 50 |
| O | 20 |
| N | 14 |
| H | 8 |
| P | 3 |
| S | 1 |
| K | 1 |
| Na | 1 |
| Ca | 0.5 |
| Mg | 0.5 |
| Cl | 0.5 |
| Fe | 0.2 |
| others | 0.3 |

Nearly half of the dry matter in cells is carbon and the elements carbon, oxygen, nitrogen and hydrogen total up to about 92% of the total.

This observation for E. coli is also found to be generally true for other cellular organisms.

Table 2 Elemental Composition of Microorganisms

| Microorganism | Carbon Source | Growth Rate | Composition |  |  |  | Empirical Formula | Molecular Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C | H | N | O |  |  |  |
| Klebsiella aerogenes | Glycerol | 0.1 | 50.6 | 7.3 | 13.0 | 29.0 | CH1.74 O0.43 N0.22 |  | 23.7 |
| Aerobacter aerogenes | Complex |  | 48.7 | 7.3 | 13.9 | 21.1 | CH1.78 O0.33 N0.24 |  | 22.5 |
| Aerobacter aerogenes | Complex | 0.9 | 50.1 | 7.3 | 14.0 | 28.7 | CH1.73 O0.24 N0.43 |  | 24.0 |
| Saccharomyces cerevisiae |  |  | 47.0 | 6.5 | 7.5 | 31.0 | CH1.66 O0.49 N0.13 |  | 23.5 |
| Sacharomyces cervisiae |  |  | 50.3 | 7.4 | 8.8 | 33.5 | CH1.75 O0.15 N0.5 |  | 23.9 |
| Candida utilis | Glucose | 0.45 | 46.9 | 7.2 | 10.9 | 35.0 | CH1.84 O0.56 N0.2 |  | 25.6 |
| Candida utilis | Ethanol | 0.43 | 47.2 | 7.3 | 11.0 | 34.6 | CH1.84 O0.55 N0.2 |  | 25.5 |

Table 2 above shows that in different microbes, the carbon content varies from 46 to 50%, hydrogen from 6 to 7%, nitrogen 8 to 14% and oxygen from 29 to 35%. These are small variations and the variations appear to depend on substrate and growth conditions. For many engineering calculations, it is reasonable to consider cell as a chemical species having the formula

This engineering approximation is a good starting point for many quantitative analysis while a more carefully formulated empirical formula based on proximate analysis may be necessary for complete material flow analysis. The cell molecular weight for the above cell formula is 12+1.8 + 0.5(16) +0.2 (14) = 24.6.

EXAMPLE 1

Suppose we want to produce 10 g of cells using glucose as a carbon source. What is the minimum amount of glucose that would be needed?

Solution

Assume cell composition as CH1.8 O0.5 N0.2

12.1+1.1,8+16.0,5+14.0,2=24,6 g/mol

lucose is C6 H12 O6

12.6+1.12+16.6= 180 g/mol

Since glucose has 6 moles of carbon per mole of glucose,

Pozn. bez uvažovania spotreby glukózy ako zdroja energie. We have assumed that all of the carbon found in substrate (glucose) is incorporated into cell mass. This does not happen as the cell needs to "oxidize" or respire some of the carbon to produce energy for biosynthesis and maintenance of cellular metabolic machinery.

GROWTH REACTION

In the above example, we have assumed that all of the carbon found in substrate (glucose) is incorporated into cell mass. This does not happen as the cell needs to "oxidize" or respire some of the carbon to produce energy for biosynthesis and maintenance of cellular metabolic machinery.

In addition cells may produce extracellular products that accumulate in the broth. Hence we can represent growth as:

The medium is the "food" for the cell. It serves as a source for all elements needed by the cell to grow (or biosynthesis) and for product formation. The compounds carbon dioxide and water on the product side of the reaction above result from oxidation of glucose in the medium.

Since the cellular material contains C, N, P, S, K, Na, Ca, etc, the medium must be formulated to supply these elements in the appropriate form.

Since the cellular material contains C, N, P, S, K, Na, Ca, etc, the medium must be formulated to supply these elements in the appropriate form. The above growth reaction can be re-stated as

If we neglect the "others" and assign stoichiometric coefficient for each of the species in the above equation on the basis of one mole of glucose (C-source) consumed, we re-write the above as

where ammonia represents the nitrogen source. We will refer to this reaction as growth reaction.

Note that whatever nitrogen that is supplied in the medium, it is expressed as equivalent nitrogen in the form of ammonia.

Cells require nitrogen in both organic and inorganic form. It is common to supply the inorganic nitrogen as salts of ammonium ( e.g. ammonium phosphate ) while the organic nitrogen is usually supplied as amino acids or proteinous extracts which are rich in nitrogen.

In most production processes using recombinant cells, glucose is used as the carbon source.

However, in the production of low value products, less expensive carbon sources such as molasses ( $ 0.10 / lb) or corn meal ( about $ 0.12 / lb ) are used. Compare this against glucose at $ 1.00 /lb!

The growth reaction derived above is useful in interpreting laboratory data reported in the literature. Because the early work in cell growth were reported by microbiologists, it is necessary for us to learn the terms used by microbiologists to describe growth stoichiometry. We will then relate the above reaction equation to commonly reported cell properties.

CELL YIELD AND STOICHIOMETRIC COEFFICIENTS

EXAMPLE 1

Consider the experimental cell (Pseudomonas lindneri) growth data shown in Fig 2-1a, originally reported by Bauchop and Elsden.

The experiment consisted of inoculating five test tubes containing growth medium with the bacterium. Each of the test tubes contained different concentrations of carbon source - in this case glucose at levels from about 4 mM to 36 mM. The cultures were incubated anaerobically (i.e. in absence of oxygen) at growth temperature ( 30° C) for two days or until growth ceases. The resulting cells were filtered, dried and weighed.

This mass of bacteria obtained is plotted against the starting glucose concentration. The important observation illustrated by the data is the straight line relationship between carbon source concentration (reactant in chemical parlance) and the cell concentration ( product ).

Obr. 1

The slope of the line represents the amount of cells obtained per unit amount of glucose consumed.

asi zle!!?:

asi správne!!?: slope = 250 μg.ml-1 / 34,7 mM = 7,2 μg.ml-1/ mM

If we convert the above to mass basis,

asi zle!!?:

ak molova hm. glukózy je C6 H12 O6

12.6+1.12+16.6= 180 g/mol

potom:

asi správne!!?: 7,2 μg.ml-1 (biomasa) / 1 mM (glukóza)=

= 7,2 μg.ml-1 (biomasa) / 180.10-3g (glukóza)=0,04 g of cell per g of glucose in 1 ml

The above value is often called cell yield, growth yield, or yield coefficient. If you examine the growth reaction stated in the previous section, the slope (in mass units) we calculated above can be equated as follows.

In the above the numerator term contains the amount of cell created and the denominator contains the amount of substrate consumed. In other words, the measurements reported by Bauschop and Elsden enable us to calculate the stoichiometric coefficient, .

That is,

asi zle!!?

Assume cell composition as CH1.8 O0.5 N0.2

12.1+1.1,8+16.0,5+14.0,2=24,6 g/mol

ak molova hm. glukózy je C6 H12 O6

12.6+1.12+16.6= 180 g/mol

výťažkový faktor biomasy na glokóze

0,04 g of cell per g of glucose in 1 ml

α=0,04.(180/24,6)=0,29

Let us consider another set of data shown in Figure 2-1b. The cell yield depends on growth conditions. You will note that under anaerobic conditions, slope (also yield) is 58.2 g (mol substrate)-1, or 0.32 g cell (g substrate)-1 . Similarly under anaerobic conditions, yield is 22 g (mol substrate)-1, or 0.21 g cell (g substrate)-1 . Invariably, the yield under anaerobic conditions will be smaller than at aerobic conditions because the cell derives significantly more metabolic energy under aerobic conditions. It is also important to note that not all cells can grow both aerobically and anaerobically.

Obr. 2

From a practical viewpoint, an aerobic organism is preferred. This is because, the amount of product protein produced is proportional to cell amount. Higher biosynthesis is possible with aerobic cultures than with anaerobic ones.

MATHEMATICAL DEFINITION OF YIELD

Mathematically, cell yield can be defined as

where X represents change in cell concentration and S represents change in substrate concentration. The subscript X/S indicates the basis of yield - cell on the basis of substrate. This notation comes in handy when we need to calculate yield based on more than one substrate. Examining the above and comparing with growth reaction, one notes that the yield defined here corresponds to a mass-based stoichiometric coefficient.

Taking the limit of Eq(2-2) as DS approaches zero,

The absolute sign is used to eliminate the negative value of the derivative. Note that dS is negative, because substrate is consumed. Yield is always reported as a positive value.

The above definition of yield can be applied to product, P on the basis of substrate consumed. Thus,

Similarly product yield based on cell will be expressed as,

In general, yield of the species, i, based on species, j, can be calculated from

From the above it is clear that we can combine two different yields which have a common species as

EXAMPLE 2

Batley (1979) reported aerobic gowth of yeast on ethanol as:

Calculate YX/E , YX/O2, YX/NH3 on mass basis.

Solution

MW of cell = 12 +1,704 +(14) (0,149) + (16) (0,408) = 22,32

MW of ethanol = (2) (12) + 5 + 16 + 1 = 46

Yield of yeast based on ethanol of about 0,5 is consistent with the observation that roughly one half of the substrate is converted to cell mass aerobically.

If yield on a carbohydrate source is significantly less than 0,5, it is likely that medium formulation is inadequate to support good growth.

EXAMPLE 3

Yeast grown on glucose is described by

C6H10O3N - emprerická formula pre sušinu pekárenskych kvasiniek

12.6+1.10+16.3+14=144g/mol

Calculate the following for a design requiring 50 g/L of yeast in a batch reactor of 100,000 liters.

Nutirent media concentration for glucose and ammonium sulfate.

Calculate YX/S and YX/O2

Calculate total oxygen required

Determine oxygen uptake rate (g O2 L-1 h-1) when cell concentration increases at a rate of 0.7 g L-1 h-1,

Solution:

Total cell mass to be produced is = (10000 L) (50 g L-1) = 5000 kg

C6H10O3N - emprerická formula pre sušinu pekárenskych kvasiniek

12.6+1.10+16.3+14=144g/mol

amoniak NH3 = 14 + 1.3=17g/mol

Yx/NH3=0,48.144/0,48.17=8,471

Y x/O2=0,48.144/3.16.2=0,72

Síran amónny (Ammonium sulfate)

(NH4)2SO4 = (14+1.4).2+32+16.4=132g/mol

MEASUREMENT OF STOICHIOMETRIC COEFFICIENTS

For the growth reaction given in Eq(2-1), the ratio γ/b is called the respiratory quotient, often abbreviated as RQ.

It is easily measured in large scale fermentors.

In Eq(2-1), if the nature of extracellular product is known (i.e. x,y,z), then it is possible to calculate and  from experimental measurement of RQ and one other measurement.

If no significant amount of extracellular product is formed, as in simply growth processes, then only RQ or one other measurment is needed to compute stoichiometric coefficients. The example given below illustrates this idea.

EXAMPLE 4

For the reaciton equation representing E. coli growth, RQ was measured as 0.85. Calculate and .

Solution:

The solution consists of carrying out elemental balances and then solving them.

Here, we can write four elemental balances, C, H, O and N.

We have five unknowns: and and a and b.

One additional relationship is obtained from the given RQ value, thus making the problem solvable!

Rearranging the above system of algebraic equations:

Solution to the above yields

THERMODYNAMICS OF CELLULAR GROWTH

Microbial growth consists of a complex network of metabolic reactions. Coupled catabolic and anabolic reactions take place so that energy released in the former is efficiently used to drive the latter.

Anabolický proces je proces syntézny – t.j. syntetizujú sa pri ňom nové organické látky alebo sa tvoria nové štruktúry. Pri takomto procese sa využíva energia získaná z fotosyntézy, chemosyntézy alebo z katabolických procesov. Takto sa syntetizujú napr. bielkoviny, fosfolipidy (zložky bunkovej membrány), alebo DNA.

Katabolický dej je proces rozkladný – je to fáza metabolizmu, kedy sa zložité molekuly – napr. bielkoviny štiepia na jednoduchšie, menšie molekuly (v prípade bielkovín na aminokyseliny). Pri takomto deji sa uvoľňuje energia vo forme ATP (adenozíntrifosfát), ktorá sa využíva pri vyššie spomenutých anabolických dejoch. Ak produkt katabolického deja je pre organizmus škodlivý alebo neužitočný, je z organizmu vylúčený (napr. dýchacou alebo tráviacou sústavou.

PRENOS ENERGIE V BUNKE

Energia vytvorená pri katabolickom deji sa uchováva a potom ďalej odovzdáva pri anabolických dejoch. Odovzdávanie energie sa uskutočňuje prostredníctvom tzv. „makroergických zlúčenín“ – t.j. zlúčenín, ktoré majú vo svojich makroergických väzbách uloženej veľa energie. Najdôležitejším prenášačom energie v bunke je adenozíntrifosfát – ATP. Molekula ATP je zložená z dusíkatej bázy – adenínu, cukru – ribózy a troch zvyškov kyseliny fosforečnej.

ATP sa podľa potreby tvorí neustále v mitochondriách. Štiepením sa ATP premieňa na adenozíndifosfát - ADP, teda spotreba energie sa realizuje na rôznych miestach bunky. Pri tejto premene (ATP na ADP) sa uvoľňuje voľná energia - ktorá sa následne môže využiť na pohyb, udržanie teploty, rozmnožovanie, a pod., a teplo – t.j. energia vyžarujúca do okolia.

Špecifická makroergická väzba sa v molekule ATP nachádza medzi fosfátovými zvyškami.

Obr. 3. Molekula ATP (zeleným - 3 fosfátové zvyšky; červeným – ribóza; modrým – adenín.

However, some energy is always lost as heat. The purpose of this chapter is to quantify the heat release due to growth.

In large-scale processes it is necessary to remove this heat so that the culture is maintained at physiological temperature. In small reactors metabolic heat is removed quite easily, while in very large fermentors ( >10,000 liters) in which rapidly growing cells are cultivated, it is necessary to design adequate heat transfer area for heat removal.

Bioreactor temperatures must be maintained within ± 0.5 C to maintain physiologic conditions conducive to optimal growth.

HEAT RELEASE DUE TO GROWTH

Consider the growth reaction when no significant amount of extracellular product is formed. Under these conditions Eq (2-1) simplifies to

Since nitrogen consumption is usually small compared to the amount of carbon consumed, and that nitrogen does not go through oxidation (while C does !), we can approximate the above as,

Consider heat balance of this reaction using one mole of glucose consumed as the basis.

where (-∆HC) and (-∆HS) are heat of combustion per gram of cell and per gram of substrate respectively.

Rearranging

The left hand side is the amount of heat released per gram of substrate consumed and the coefficient of the first term on the right is growth yield. That is

where Y/S is "heat yield" on the basis of substrate consumed. Dividing the above by Y X/S gives

Y∆/x =                                    (3-4)

Both Eq (3-1) and (3-2) are useful in determining heat release due to growth, Y/X and substrate consumption, Y/S.

HEAT OF COMBUSTION DATA

Heat of combustion has been reported by a number of authors and a few are given below in Table 3-1. The heat of combustion for a variety of organism falls in a very narrow range of about 22 kJ g-1. Heat of combustion of glucose, a commonly used substrate, is 15.6 kJ g-1.

Table 3-1 Heat of Combustion of Biomass

| Organism | (-Hs), kJ g-1 |
| --- | --- |
| E. coli | 23.03 |
| E. cloacae | 22.83 |
| B. thuringiensis | 22.08 |
| Candida lipolytica | 21.34 |
| Candida boidinii | 20.14 |
| Kluyveromyces fragilis | 21.66 |

Experimental Observations

Cooney and co-workers collected heat release experimental data for a number of different organisms by making careful heat balance measurements over a fermentor. The data show a linear relationship between heat released and oxygen consumption rate (Fig 2-1).

Obr. 4 Figure 3-1 Experimental Heat Release Data (after Cooney et al ).

The slope of the line above is equal to the amount of heat released per mol of oxygen consumed. That is:

This result enables one to calculate heat generation rate from oxygen uptake rate information. We will examine oxygen uptake rate data in Chapter 5.

Heat Release when Extracellular Products are Formed

When significant amount of product is present, Eq(3-1) will be modified to

where (-Hp) is heat of combustion per gram of extracellular product(s). Dividing the above by MW of substrate gives

Dividing the above by gives

EXAMPLE 5

In citric acid fermentation, 2000 kg glucose is fermented over a 50 h period resulting in 1300 kg citric acid and 400 kg biomass.

The process required 800 kg O2.

Calculate the total heat released. Heat of combustion of citric acid (C6H8O7) is -1,960 kJ gmol-1.

citric acid (C6H8O7) = 12.6+1.8+16.7=192

Solution

This is a case of extracellular product is formed.

Use Eq (3-6) or (3-7) to first obtain heat yield.

(-HC) is heat of combustion per gram of cell

(-HS) is heat of combustion per gram of substrate

(-Hp) is heat of combustion per gram of extracellular product

pozor chyba! namiesto 4000 má byť 400, tak ako v zadaní!

The negative value indicates that heat is rejected or being generated due to growth.

The interpretation here is similar to that for a chemical reaction. Recall heat of reaction is negative for an exothermic reaction. Ignoring the sign as we label it as heat generated,

Alternatively: we can calculate heat generated from oxygen uptake data.

The two methods differ by about 30%, assuming the second method is more accurate!

In the first method, it is assumed that all the extracellular products are accounted for in citric acid. It is likely that the cell produces other products that are not included in the energy balance.

PROCESOVÉ CHARAKTERISTIKY BIOREAKTORA

Spravte experimenty na laboratórnom vsádzkovom bioreaktore s objemom 6 litrov s modelovými kvapalinami, s cieľom preskúmať vzájomný vplyv jednotlivých parametrov charakterizujúcich procesy v bioreaktoroch. Ako nezávisle premenné parametre uvažujte otáčky miešadla a prietok vzduchu. Závisle premenné parametre sú teplota, pH, dO2, dCO2, Redox. Vyhodnoďte vzájomný vplyv jednotlivých parametrov formou grafov a textovým komentárom vychádzajúcim z dostupných poznatkov v oblasti procesného strojníctva.

Obr. 5. Bioreaktor Esedra 6 Plus.

Esedra Plus 6.0 od firmy Solaris je bioreaktor pracujúci vsádzkovo, ktorý je možné použiť na kultiváciu buniek ale aj mikroorganizmov.

Nádoba ako aj duplikátor tohto bioreaktora sú vyrobené zo skla, zvyšné časti sú vyrobené z nerezovej oceli.

Vo vnútri nádoby sa nachádza hriadeľ s miešadlom a narážkami, ktoré zabraňujú vytvoreniu

centrálneho víru, ktorý môže vzniknúť pôsobením miešadla.

Vo veku sú procesné otvory, cez ktoré sú do nádoby zavedené sondy. Sondy majú za úlohu merať a zaznamenávať procesné parametre. . Bioreaktor má 5 sond, ktoré merajú teplotu, dCO2, dO2, pH a Redox.

Z vonkajšej strany nádoby sa nachádza duplikátor, ktorý v prípade potreby znižuje teplotu náplne. Ohrev náplne zabezpečuje ohrevné teleso, ktoré je zabudované zospodu bioereaktora.

Prísun kyslíka zabezpečuje kompresor, ktorý doň privádza vzduch z okolitého prostredia.

Aby bola dosiahnutá úplna sterilita, prichádzajúci vzduch prechádza cez mikrobiálne filtre, ktoré ho zbavujú nežiaducich mikroorganizmov. Rovnako aj výstup z bioreaktora je opatrený

mikrobiálnym filtrom, ktorý zabraňuje vniknutiu mikroorganizmov do náplne v prípade, že by

sa v biorektore vytvoril podtlak.

Sondy bioreaktora sú napojené na riadiacu jednotku, ktorá dokáže meranie vyhodnotiť a poskytnúť výstup vo forme grafu.

OTÁZKY NA SKÚŠKU:

Pre túto kapitolu nie sú.